EXP NO: 17

UART INTERFACING USING INTERRUPT IN KEIL

AIM:

To receive serial data using interrupt.

APPARATUS REQUIRED:

• PC with Windows OS

• Keil μVision IDE

• 8051 Simulator (Keil built-in)

• Serial Window (UART #1)

PROGRAM:

#include <reg51.h>

void serial_ISR(void) interrupt 4

{

if(RI)

{

P1 = SBUF;

RI = 0;

}

}

void main()

{

TMOD=0x20;

TH1=0xFD;

SCON=0x50;

TR1=1;

ES=1;

EA=1;

while(1);

}

OUTPUT:

OBSERVATIONS:

Input: Character typed in UART window

Processing: Serial interrupt triggered

Output: Port 1 displays ASCII value of received character

Typed ASCII P1

A 41H 0x41

a 61H 0x61

1 31H 0x31

Received serial data appears on Port1.

RESULT:

Thus the serial data was received using Interrupt and the received data was verified.