EXP NO: 11

STUDY OF GPIO & BIT ADDRESSING IN AT89C51

AIM:

To study GPIO control and bit-addressable SFR using Embedded C.

SOFTWARE REQUIRED:

Keil

THEORY

8051 ports are byte and bit addressable. Using sbit, individual bits of a port can be accessed

directly. Ports default to logic HIGH due to internal pull-ups.

PROGRAM

#include <reg51.h>

sbit LED1 = P1^0;

sbit LED2 = P1^1;

void delay()

{

unsigned int i,j;

for(i=0;i<200;i++)

for(j=0;j<1000;j++);

}

void main()

{

while(1)

{

LED1 = 1;

LED2 = 0;

delay();

LED1 = 0;

LED2 = 1;

delay();

}

}

OUTPUT:

OBERVATION:

Register Value

P1 0x01

P1 0x02

RESULT:

Thus GPIO and bit addressing was studied and in Port1 the pins P1.0 and P1.1 toggled

alternately.